# Applications

- Electric vehicles

- Drones

- Energy storage

- Research programs

# Product summary:

ENNOID-BMS is a configurable battery management system consisting of a Master board based on an STM32 microcontroller connected through an ISOSPI interface to several modular slave boards. ENNOID-BMS can monitor the specifics temperatures, currents & voltages that are critical for any lithium-ion battery packs. Based on the monitored inputs & the configured parameters, the master board can allow or interrupt the flow of energy from the battery pack by switching the state of external heavy-duty contactors. ENNOID-BMS can measure each cell voltage level & can trigger the passive balancing function during charging for cells above the configured limit to ensure that all cells have a similar State-Of-Charge (SOC). Parameters can be configured through the ENNOID-BMS-Tool software running on a USB connected host computer.

# Features

- Up to 400V operation

- Up to 500A continuous operation

- Modular with master/slave topology

- Isolation between battery pack & components

- Communication between slaves & master through a two-wire daisy chained ISOSPI interface

- Isolated CAN bus interface

- Isolated charger detection circuit

- Isolated voltage measurement for battery & load

- Isolated bi-directional current monitoring via external delta-sigma current sensor

- 12V contactor coil outputs for charge, discharge & pre-charge circuits

- USB interface for programming and firmware upgrades through an easy to use graphical user interface

- OLED Display & power button

- 2.5V to 4.5V cell voltage operation

# Block diagram

Figure 1: Simplified block diagram

# Pinout information

## Master board

The master board is equipped with an STM32F3 microcontroller which enables all functionalities of the battery management system.

4

1

2

3

11

5

6

7

8

9

10

14

13

12

Figure 2 : Master board

| # | Name | Description |  |
| --- | --- | --- | --- |
| 1 | ExtPower | + | 12V DC power input |
|  |  | - | ISOGND |
| 2 | Discharge | + | 12V DC output for discharge contactor |
|  |  | - | ISOGND |
| 3 | Precharge | + | 12V DC output for precharge contactor |
|  |  | - | ISOGND |
| 4 | Charge | + | 12V DC output for charge contactor |
|  |  | - | ISOGND |
| 5 | - Discharge + | + | 0-400V isolated DC input for load voltage measurement |
|  |  | - | BATGND |
| 6 | - BAT + | + | 0-400V isolated DC input for battery voltage measurement |
|  |  | - | BATGND |
| 7 | - Charge + | + | 0-400V isolated DC input for charger detection |
|  |  | - | BATGND |
| 8 | Current | 3.3V | +3.3V |
|  |  | I+ | Delta-sigma signal from isolated current sensor |
|  |  | I- | Not connected |
|  |  | GND | ISOGND |
| 9 | Shunt1* | 3.3V | +3.3V |
|  |  | I+ | Differential input + from isolated differential current sensor |
|  |  | I- | Differential input - from isolated differential current sensor |
|  |  | GND | ISOGND |
| 10 | Master | PA | Isolated ISOSPI communication 2 wire interfaces with slave boards |
|  |  | MA | Isolated ISOSPI communication 2 wire interfaces with slave boards |
| 11 | USB | Micro-USB interface with ENNOID-BMS-tool software on a computer |  |
| 12 | CAN | EN | External enable signal |
|  |  | 5V | External 5V for CAN |
|  |  | H | CANH |
|  |  | L | CANL |
|  |  | GND | CANGND |
| 13 | OLED | 3.3V | +3.3V |
|  |  | RST | No connect |
|  |  | SDA | SDA signal output for OLED display |
|  |  | SCL | SCL signal output for OLED display |
|  |  | GND | ISOGND |
| 14 | Power Button | GND | ISOGND |
|  |  | N.O. | Normally open pin for power button |
|  |  | 3.3V | No connect +3.3V |
|  |  | LED | +3.3V for LED (optional) |
|  |  | GND | ISOGND |
|  | Debug** | +3.3V |  |
|  |  | SWCLK |  |
|  |  | ISOGND |  |
|  |  | SWDIO |  |
|  |  | NRST |  |

* Current & shunt1 ports requires different components for operating. User must choose which one is appropriate.

**Debug port is not required for operation

## Slave board LTC6811

The slave board is using an LTC6811 IC. It can monitor up to 12 cells in series. The slave board is powered directly by the lithium-ion cells it monitors.

1

2

3

7

6

5

4

Figure 3: Slave board

| # | Name | Description |  |
| --- | --- | --- | --- |
| 1 | Master | Isolated ISOSPI communication 2 wire interfaces towards previous slave board or master board |  |
| 2 | Slave | Isolated ISOSPI communication 2 wire interfaces towards next slave board |  |
| 3 | LAST? | Leave pins unconnected for the last slave board in the battery pack. Otherwise, those pins must be shorted. |  |
| 4 | Cells | BAT- | Connect to negative voltage of the battery module |
|  |  | 1 to 12 | Connect all subsequent pins to the next cells positive voltage levels. |
| 5 | Analog* | Analog input that can be accessed & sampled if needed (temperature sensor) |  |
| 6 | I2C* | I2C communication channel for 8 additional temperature sensors through external ADC128 IC |  |
| 7 | GPIO* | GPIO for external signal if needed |  |

* Not mandatory for operation

# Other related hardware

## Current sensor

We recommend using ENNOID dedicated Delta-Sigma Current sensor board

## Power supply

ENNOID-BMS require a 12V power supply which can supply power for the Master-BMS board. A peak current of 3A is required for a few milliseconds for closing most type of high current external contactors. Only a few milliamps are required afterward closing. The power supply must be able to handle contactors peak loads.

We recommend using our ENNOID-BMS 12V DC output power supply board which is fed and isolated from the battery pack itself. We offer two versions with different voltage inputs:

- 24V to 150V

- 75V to 400V

## OLED display

SSD1306 compatible OLED display is not mandatory for operating ENNOID-BMS but is recommended.

## Power button

Power button N.O. type toggle switch is required for turning ON/OFF ENNOID-BMS. External activation of ENNOID-BMS is possible through CAN bus or with USB.

## Contactor

Contactor with economizer is recommended. Some contactors with high coil resistance don’t require economizer. ENNOID-BMS does not actually support PWM outputs for contactors without build-in economizer. LEV200, EV200, EVC250 or EVC500 contactors with economizer are recommended

# Software

Download ENNOID-BMS-Tool software for windows:

https://github.com/EnnoidMe/ENNOID-BMS-Tool/releases

# ENNOID-BMS configuration

- Connect at minimum a 12V power supply to ENNOID-BMS Master board.

- Connect a host computer running the ENNOID-BMS-Tool to the ENNOID-BMS master board with a micro USB cable. (Warning: some cheap USB cables cannot carry data and won’t work with ENNOID-BMS) Power LED indicator should light up. Click connect on the connection page. The app should now shows “connected” in the right side bottom corner.

- Go into firmware tab and upload latest firmware (this step is needed only for initial setup or uploading a new firmware version)

- Under “Master Settings” tab, user can define all parameters of the BMS (Tabs under “Slave Settings” are not required for configuring ENNOID-BMS). Modified parameters can be read, applied & saved by clicking on the associated buttons on the right-side panel.

- For real-time data logging & testing the BMS, the “Data Analysis” tab shows in real time the measured pack voltage, load voltage, current, Temperatures, BMS status & all cells voltages. You need to click on the “RT” button on the left side of the screen to activate real time communication.

- Once properly configured, you can operate the ENNOID-BMS with the ON/OFF power button, & the OLED display will show your battery status.

Enjoy!