Figure 1 : Master board

| # | Name | Description |  |
| --- | --- | --- | --- |
| 1 | ExtPower | + | 12V DC power input |
|  |  | - | ISOGND |
| 2 | Discharge | + | 12V DC output for discharge contactor |
|  |  | - | ISOGND |
| 3 | Precharge | + | 12V DC output for precharge contactor |
|  |  | - | ISOGND |
| 4 | Charge | + | 12V DC output for charge contactor |
|  |  | - | ISOGND |
| 5 | - Discharge + | + | 0-400V DC input for load voltage measurement |
|  |  | - | BATGND |
| 6 | - BAT + | + | 0-400V DC input for battery voltage measurement |
|  |  | - | BATGND |
| 7 | - Charge + | + | 0-400V DC input for charger detection |
|  |  | - | BATGND |
| 8 | Current | 3.3V | +3.3V |
|  |  | I+ | Delta-sigma signal from current sensor |
|  |  | I- | Not connected |
|  |  | GND | ISOGND |
| 9 | Shunt1* | 3.3V | +3.3V |
|  |  | I+ | Differential input + from differential current sensor |
|  |  | I- | Differential input - from differential current sensor |
|  |  | GND | ISOGND |
| 10 | Master | PA | Isolated ISOSPI communication 2 wire interfaces with slave boards |
|  |  | MA | Isolated ISOSPI communication 2 wire interfaces with slave boards |
| 11 | USB | Micro-USB interface with ENNOID-BMS-tool software on a computer |  |
| 12 | CAN | EN | External enable signal |
|  |  | 5V | External 5V for CAN |
|  |  | H | CANH |
|  |  | L | CANL |
|  |  | GND | CANGND |
| 13 | OLED | 3.3V | +3.3V |
|  |  | RST | No connect |
|  |  | SDA | SDA signal output for OLED display |
|  |  | SCL | SCL signal output for OLED display |
|  |  | GND | ISOGND |
| 14 | Power Button | GND | ISOGND |
|  |  | N.O. | Normally open pin for power button |
|  |  | 3.3V | No connect +3.3V |
|  |  | LED | +3.3V for LED |
|  |  | GND | ISOGND |
| 15 | Debug | +3.3V |  |
|  |  | SWCLK |  |
|  |  | ISOGND |  |
|  |  | SWDIO |  |
|  |  | NRST |  |

* Current & shunt1 ports requires different components for operating. User must choose which one is appropriate.

Figure 2: Slave board

| # | Name | Description |  |
| --- | --- | --- | --- |
| 1 | Master | Isolated ISOSPI communication 2 wire interfaces towards previous slave board or master board |  |
| 2 | Slave | Isolated ISOSPI communication 2 wire interfaces towards next slave board |  |
| 3 | LAST? | Leave pins unconnected for the last slave board in the battery pack. Otherwise, those pins must be shorted. |  |
| 4 | Cells | BAT- | Connect to negative voltage of the battery module |
|  |  | 1 to 12 | Connect all pins to the next cells positive voltage levels. |
| 5 | Analog | Analog input that can be accessed & sampled if needed |  |
| 6 | I2C | I2C communication channel for 8 temperature sensors through external ADC128 IC |  |
| 7 | GPIO | GPIO for external signal if needed |  |