# Applications

- Electric vehicles

- Drones

- Energy storage

- Research programs

# Product summary:

Lithium-ion battery packs requires protection against temperatures, currents & voltages abuses during operation. ENNOID-BMS can accurately monitor temperatures, currents & voltages of lithium-ion battery packs and can allow or interrupt the flow of energy at any moment by switching externals heavy duty contactors.  Furthermore, ENNOID-BMS can measure each cell voltage level & trigger the balancing function during the charging process to ensure that all cells have a similar State-Of-Charge (SOC) & voltages. Parameters for balancing, temperature, currents & voltages can be configured by the user through the ENNOID-BMS-Tool software.

# Features

- Modular with master/slave topology

- Isolation between battery pack & components

- Communication between slaves & master through a two-wire daisy chained ISOSPI interface

- Isolated CAN bus interface

- Charger detection circuit

- Isolated voltage measurement for Battery & load voltages

- Isolated bi-directional current monitoring via external delta-sigma current sensor

- 12-24V contactor coil outputs for charge, discharge & precharge

- USB interface for programming and firmware upgrades through an easy to use graphical user interface

- OLED Display & power button

# Block diagram

Figure 1: Simplified block diagram

# Pinout information

## Master board

4

1

2

3

5

6

7

8

9

10

14

13

12

11

Figure 2 : Master board

| # | Name | Description |  |
| --- | --- | --- | --- |
| 1 | ExtPower | + | 12V DC power input |
|  |  | - | ISOGND |
| 2 | Discharge | + | 12V DC output for discharge contactor |
|  |  | - | ISOGND |
| 3 | Precharge | + | 12V DC output for precharge contactor |
|  |  | - | ISOGND |
| 4 | Charge | + | 12V DC output for charge contactor |
|  |  | - | ISOGND |
| 5 | - Discharge + | + | 0-400V DC input for load voltage measurement |
|  |  | - | BATGND |
| 6 | - BAT + | + | 0-400V DC input for battery voltage measurement |
|  |  | - | BATGND |
| 7 | - Charge + | + | 0-400V DC input for charger detection |
|  |  | - | BATGND |
| 8 | Current | 3.3V | +3.3V |
|  |  | I+ | Delta-sigma signal from current sensor |
|  |  | I- | Not connected |
|  |  | GND | ISOGND |
| 9 | Shunt1* | 3.3V | +3.3V |
|  |  | I+ | Differential input + from differential current sensor |
|  |  | I- | Differential input - from differential current sensor |
|  |  | GND | ISOGND |
| 10 | Master | PA | Isolated ISOSPI communication 2 wire interfaces with slave boards |
|  |  | MA | Isolated ISOSPI communication 2 wire interfaces with slave boards |
| 11 | USB | Micro-USB interface with ENNOID-BMS-tool software on a computer |  |
| 12 | CAN | EN | External enable signal |
|  |  | 5V | External 5V for CAN |
|  |  | H | CANH |
|  |  | L | CANL |
|  |  | GND | CANGND |
| 13 | OLED | 3.3V | +3.3V |
|  |  | RST | No connect |
|  |  | SDA | SDA signal output for OLED display |
|  |  | SCL | SCL signal output for OLED display |
|  |  | GND | ISOGND |
| 14 | Power Button | GND | ISOGND |
|  |  | N.O. | Normally open pin for power button |
|  |  | 3.3V | No connect +3.3V |
|  |  | LED | +3.3V for LED |
|  |  | GND | ISOGND |
|  | Debug** | +3.3V |  |
|  |  | SWCLK |  |
|  |  | ISOGND |  |
|  |  | SWDIO |  |
|  |  | NRST |  |

* Current & shunt1 ports requires different components for operating. User must choose which one is appropriate.

**Debug port is not required for operation

## Slave board

1

2

3

7

6

5

4

Figure 3: Slave board

| # | Name | Description |  |
| --- | --- | --- | --- |
| 1 | Master | Isolated ISOSPI communication 2 wire interfaces towards previous slave board or master board |  |
| 2 | Slave | Isolated ISOSPI communication 2 wire interfaces towards next slave board |  |
| 3 | LAST? | Leave pins unconnected for the last slave board in the battery pack. Otherwise, those pins must be shorted. |  |
| 4 | Cells | BAT- | Connect to negative voltage of the battery module |
|  |  | 1 to 12 | Connect all pins to the next cells positive voltage levels. |
| 5 | Analog* | Analog input that can be accessed & sampled if needed |  |
| 6 | I2C | I2C communication channel for 8 temperature sensors through external ADC128 IC |  |
| 7 | GPIO* | GPIO for external signal if needed |  |

* Not mandatory for operation

# Software:

Download ENNOID-BMS-Tool software for windows: https://github.com/EnnoidMe/ENNOID-BMS-Tool/releases

# ENNOID-BMS configuration

- Connect to ENNOID-BMS with a micro USB cable. (Be careful, some cheap USB cables cannot carry data and won’t work with ENNOID-BMS)

- Go into firmware tab and upload latest firmware (this step is needed only for initial setup or uploading a new firmware version)

- In “Master Settings” tab, user can define all parameters of the BMS (“Slave Settings” tab is not required for configuring ENNOID-BMS)

- For real-time data logging & testing the BMS, the “Data Analysis” tab shows in real time the measured pack voltage, load voltage, current, Temperatures, BMS status & all cells voltages. You need to click on the “RT” button on the left side of the screen to activate real time communication.

- Once properly configured, you can operate the ENNOID-BMS with the ON/OFF power button, & the OLED display will show your battery status.

Enjoy!